# M12 8-Pin Connector – Pin Assignment

Recommended industrial pin assignment for an optical sensor system with 24V supply, digital status outputs and one 4–20mA analog output.

## Pin Assignment

| Pin | Signal | Description |
| --- | --- | --- |
| 1 | +24V DC | Sensor power supply |
| 2 | Sensor Active | Sensor ready output |
| 3 | GND | Ground / 0V |
| 4 | Glass In Sensor | Glass detected output |
| 5 | Coating Front | Front coating detected |
| 6 | Coating Back | Back coating detected |
| 7 | 4–20mA Thickness (1-25mm) | Glass thickness analog output |
| 8 | Reserved | Reserved for future use |

## Connector Front View (M12 A-coded, 8-pin)

## Signal Description

+24V DC: 24V supply voltage for the sensor system

Sensor Active: Digital output HIGH when sensor is operational

GND: Ground / reference potential

Glass In Sensor: Digital output HIGH when glass is detected

Coating Front: Digital output HIGH when front coating is detected

Coating Back: Digital output HIGH when back coating is detected

4–20mA Thickness: Analog output proportional to glass thickness

Reserved: Unused / reserved for future extensions

## Recommendations

Use a shielded 8-core cable

Use a twisted pair for the 4–20mA signal if possible

Connect cable shield 360° to the metal housing

Use 24V PNP outputs for digital signals

Keep the reserved pin available for future extensions

## Technical Data

| Connector | M12 A-coded, 8-pin |
| --- | --- |
| Supply Voltage | 24V DC ±10% |
| Digital Outputs | PNP 24V |
| Analog Output | 4–20mA (1-25mm) |
| Protection Class | IP54/IP65 depending on enclosure |